Workshop on Analog-to-Digital Conversion and Datalogging tasks in MicroPython and ESP32 (Percentage of Final Grades: 20%)

You have been hired to implement the alarm system for a baby incubator with triple temperature redundancy, which has the following inputs and outputs:

- Temperature sensor: An LM35-based sensor that has to be connected to an ADC input

- Voltage monitoring: A voltage monitoring system using a potentiometer and an ADC input must also be included from 0 to 3.3 VDC.

- Thermocouple: A MAX6675 + sensor must be connected via SPI to the microcontroller.

- Barometric pressure + temperature: A BMP280 must be connected to the microcontroller via I2C.

- Normal temperature indicator: a green LED must be considered and activated if the temperature is lower than a threshold value called warning_level (configurable).

- Warning temperature indicator: a yellow LED must be considered and activated if the temperature is greater than warning_level and lower than another level called emergency_level (configurable).

- Emergency temperature indicator: a red LED must be considered if the temperature exceeds emergency_level.

- Low voltage indicator: another red LED must be considered if the voltage is less than a threshold value. It helps technicians detect when the baby incubator has electrical problems.

The user should be able to choose which of the three temperature sensors is used for the temperature indicators, i.e., thermocouple, LM35, or BMP280. In addition, all sensor values must be transmitted and displayed on the PC using the UART interface.

A MicroPython-based software must be coded with the following features:

- A generic class that allows reading any ADC-based input. This class must be used for two instances: the LM35 sensor and the voltage meter. This class must run in parallel to the main program (25%).

- A class that drives the indicator LEDs for normal, emergency, and warning temperature levels. The configuration must be read from a JSON file (25%).

- A class that sends the information of all sensors to the PC via UART (25%).

The remaining 25% will be the wiring clean code.

This assignment will be graded on May 13th during class time. No prorogations allowed.